Soil Moisture Monitoring System

Using Arduino Uno

Project Documentation

Arduino-Based Soil Moisture Monitoring and Alert System

# 1. Introduction

The Arduino-Based Soil Moisture Monitoring and Alert System is an embedded systems project designed to measure the moisture content of soil using two metal probes connected to an Arduino Uno. The system continuously monitors the soil moisture level and provides both visual and audible indications of the soil's condition.

When the soil is wet, a blue LED lights up to indicate sufficient moisture. When the soil becomes dry, a red LED and a buzzer are activated to alert the user that the soil requires watering.

This project introduces students to analog sensors, Arduino programming, voltage divider circuits, and basic automation concepts used in modern smart agriculture.

# 2. Objectives

- Measure the moisture level of soil using metal probes

- Indicate wet soil condition using a blue LED (D8)

- Indicate dry soil condition using a red LED (D9)

- Activate a buzzer (D10) when the soil becomes dry

- Output real-time moisture readings via Serial Monitor at 9600 baud

- Understand analog sensor interfacing and ADC conversion on Arduino

- Learn basic decision-making using if/else conditional programming

# 3. Learning Outcomes

After completing this project, students will be able to:

- Understand the working principle of a resistive soil moisture sensor

- Understand how analogRead() converts voltage to a 0–1023 integer value

- Use pinMode() and digitalWrite() to control LEDs and buzzers

- Use Serial.begin() and Serial.println() for live sensor output

- Apply threshold-based conditional logic to control hardware outputs

- Build a basic smart agriculture monitoring system from scratch

# 4. Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| Soil Moisture Probes (Metal) | 1 Pair |
| 10 kΩ Resistor | 1 |
| Blue LED | 1 |
| Red LED | 1 |
| 220 Ω Resistors | 2 |
| Active Buzzer | 1 |
| Breadboard | 1 |
| Jumper Wires | Several |
| USB Cable | 1 |
| 9V Battery (Optional) | 1 |
| Small Enclosure Box (Optional) | 1 |

# 5. Estimated Cost

| Item | Approx. Cost (NPR) |
| --- | --- |
| Arduino Uno | 1,800 |
| Soil Probes | 200 |
| LEDs | 50 |
| Active Buzzer | 100 |
| Breadboard | 250 |
| Jumper Wires | 200 |
| Resistors | 20 |
| Battery & Clip | 300 |
| Enclosure | 500 |

Estimated Total Cost: NPR 3,500 – 5,000

# 6. Working Principle

The soil moisture sensor consists of two metal probes inserted into the soil. Water in the soil conducts electricity, reducing resistance between the probes. Dry soil has high resistance, resulting in less current flow.

The probes and a 10 kΩ pull-up resistor form a voltage divider connected to analog pin A0. The Arduino's ADC converts the voltage to an integer between 0 and 1023.

Threshold Logic (from sketch)

The firmware compares the ADC reading against a configurable threshold of 500:

| ADC Reading | Soil Condition | Blue LED (D8) | Red LED (D9) | Buzzer (D10) |
| --- | --- | --- | --- | --- |
| 0 – 499 | Wet Soil (low resistance) | HIGH (ON) | LOW (OFF) | LOW (OFF) |
| 500 – 1023 | Dry Soil (high resistance) | LOW (OFF) | HIGH (ON) | HIGH (ON) |

The threshold value (500) can be adjusted in the sketch after testing with your specific soil type. A comment in the code marks the exact line: "// Adjust this after testing".

# 7. Circuit Diagram

Voltage Divider — Moisture Sensor

+5V

|

10kΩ

|

+-----------> A0 (soilPin)

|

Soil Probe 1

|

(Soil)

|

Soil Probe 2

|

GND

LED & Buzzer Connections

Blue LED:  D8  ----[220Ω]----|>|---- GND

Red LED:   D9  ----[220Ω]----|>|---- GND

Buzzer:    D10 --------[Buzzer]------ GND

# 8. Pin Connections

| Arduino Pin | Variable in Code | Connected Component |
| --- | --- | --- |
| A0 | soilPin | Soil moisture voltage divider output |
| D8 | blueLED | Blue LED (via 220 Ω resistor) |
| D9 | redLED | Red LED (via 220 Ω resistor) |
| D10 | buzzer | Active buzzer |
| 5V | — | Voltage divider top rail |
| GND | — | Common ground for all components |

# 9. Source Code

The complete Arduino sketch is shown below. It uses only built-in Arduino functions — no external libraries are required.

Pin & Threshold Declarations

Four constants are declared globally. The threshold constant is intentionally isolated and commented for easy calibration:

const int soilPin = A0;

const int blueLED = 8;

const int redLED  = 9;

const int buzzer  = 10;

// Adjust this after testing

const int threshold = 500;

setup() Function

Configures D8, D9, and D10 as digital outputs and opens Serial communication at 9600 baud for live monitoring:

void setup() {

pinMode(blueLED, OUTPUT);

pinMode(redLED,  OUTPUT);

pinMode(buzzer,  OUTPUT);

Serial.begin(9600);

}

loop() Function

Runs continuously. Reads the ADC value from A0, prints it to Serial Monitor, then applies the threshold condition to drive the LEDs and buzzer. A 500ms delay paces the readings:

void loop() {

int soilValue = analogRead(soilPin);

Serial.print("Soil Moisture: ");

Serial.println(soilValue);

if (soilValue >= threshold) {

// Soil is dry

digitalWrite(blueLED, LOW);

digitalWrite(redLED,  HIGH);

digitalWrite(buzzer,  HIGH);

} else {

// Soil is wet

digitalWrite(blueLED, HIGH);

digitalWrite(redLED,  LOW);

digitalWrite(buzzer,  LOW);

}

delay(500);

}

Full Sketch

Complete sketch for copy/paste or direct upload to Arduino IDE:

const int soilPin = A0;

const int blueLED = 8;

const int redLED  = 9;

const int buzzer  = 10;

// Adjust this after testing

const int threshold = 500;

void setup() {

pinMode(blueLED, OUTPUT);

pinMode(redLED,  OUTPUT);

pinMode(buzzer,  OUTPUT);

Serial.begin(9600);

}

void loop() {

int soilValue = analogRead(soilPin);

Serial.print("Soil Moisture: ");

Serial.println(soilValue);

if (soilValue >= threshold) {

// Soil is dry

digitalWrite(blueLED, LOW);

digitalWrite(redLED,  HIGH);

digitalWrite(buzzer,  HIGH);

} else {

// Soil is wet

digitalWrite(blueLED, HIGH);

digitalWrite(redLED,  LOW);

digitalWrite(buzzer,  LOW);

}

delay(500);

}

# 10. Code Walkthrough

| Line / Block | Purpose |
| --- | --- |
| const int soilPin = A0 | Maps analog pin A0 to a readable variable name |
| const int blueLED = 8 | Maps digital pin D8 — controls the wet indicator LED |
| const int redLED = 9 | Maps digital pin D9 — controls the dry indicator LED |
| const int buzzer = 10 | Maps digital pin D10 — controls the active buzzer |
| const int threshold = 500 | Calibration constant — change this to suit your soil type |
| Serial.begin(9600) | Opens serial port at 9600 baud for live sensor output |
| analogRead(soilPin) | Reads A0 voltage and returns an integer 0–1023 |
| Serial.print / println | Sends moisture value to Serial Monitor each loop cycle |
| if (soilValue >= threshold) | Dry condition branch — activates red LED and buzzer |
| else | Wet condition branch — activates blue LED, silences alarm |
| delay(500) | Pauses 500ms between readings for sensor stability |

# 11. Algorithm

| Step | Action |
| --- | --- |
| 1 | Power on — Arduino initializes |
| 2 | setup(): set D8, D9, D10 as OUTPUT; open Serial at 9600 baud |
| 3 | loop(): call analogRead(A0) → store in soilValue (0–1023) |
| 4 | Serial.print("Soil Moisture: ") + Serial.println(soilValue) |
| 5 | Evaluate: soilValue >= threshold (500)? |
| 6a | YES (dry): digitalWrite(blueLED, LOW), digitalWrite(redLED, HIGH), digitalWrite(buzzer, HIGH) |
| 6b | NO (wet): digitalWrite(blueLED, HIGH), digitalWrite(redLED, LOW), digitalWrite(buzzer, LOW) |
| 7 | delay(500) — wait 500 ms |
| 8 | Return to Step 3 — repeat indefinitely |

# 12. Program Logic Summary

| soilValue | Condition | blueLED (D8) | redLED (D9) | buzzer (D10) |
| --- | --- | --- | --- | --- |
| 0 – 499 | Wet Soil | HIGH | LOW | LOW |
| 500 – 1023 | Dry Soil | LOW | HIGH | HIGH |

# 13. Software Requirements

- Arduino IDE (version 1.8.x or 2.x)

- No external libraries required — uses only built-in Arduino functions

- Serial Monitor (built into Arduino IDE) — set to 9600 baud to view output

Uploading the Sketch

- Connect Arduino Uno to PC via USB cable

- Open Arduino IDE, paste the full sketch

- Select board: Tools → Board → Arduino Uno

- Select port: Tools → Port → (your COM or /dev/tty port)

- Click Upload (Ctrl+U)

- Open Serial Monitor (Ctrl+Shift+M), set baud to 9600

# 14. Threshold Calibration

The default threshold of 500 may not suit every soil type. To calibrate:

- Upload the sketch and open the Serial Monitor at 9600 baud

- Insert probes into fully wet soil — note the ADC value displayed

- Allow the soil to dry completely — note the ADC value again

- Set the threshold to a value midway between the two observed readings

- Change the value in the sketch: const int threshold = <your value>;

- Re-upload and verify the system switches correctly between wet and dry

Example: if wet soil reads ~150 and dry soil reads ~820, a threshold of 500 is appropriate. If wet reads ~300, raise the threshold to ~600.

# 15. Expected Output

Wet Soil (soilValue = 180)

| Output | State |
| --- | --- |
| Serial Monitor | Soil Moisture: 180 |
| Blue LED (D8) | ON (HIGH) |
| Red LED (D9) | OFF (LOW) |
| Buzzer (D10) | OFF (LOW) |

Dry Soil (soilValue = 785)

| Output | State |
| --- | --- |
| Serial Monitor | Soil Moisture: 785 |
| Blue LED (D8) | OFF (LOW) |
| Red LED (D9) | ON (HIGH) |
| Buzzer (D10) | ON (HIGH) |

Readings update every 500 milliseconds as controlled by delay(500) in the loop.

# 16. Features

- Real-time soil moisture monitoring via analogRead() every 500ms

- Live Serial Monitor output at 9600 baud for calibration and debugging

- Dual LED visual feedback — blue for wet, red for dry

- Active buzzer alarm on dry soil condition

- Configurable threshold constant for different soil types

- No external libraries — minimal dependencies

- Beginner-friendly code structure with clear comments

# 17. Applications

- Smart irrigation systems

- Home gardening and plant care automation

- Agricultural field monitoring

- School and college STEM projects

- Plant watering reminder systems

- Greenhouse environment monitoring

# 18. Advantages

- Easy to build — minimal components and wiring

- Low cost (NPR 3,500 – 5,000)

- Low power consumption

- Portable and compact

- Immediate visual and audible feedback

- Easily calibrated via a single threshold constant

# 19. Limitations

- Metal probes corrode over time in moist soil — reduces accuracy

- Readings vary by soil type — threshold requires calibration per environment

- Does not automatically water plants — alert only, no actuator control

- No data logging — readings are not stored between power cycles

- No wireless output — Serial Monitor requires physical USB connection

# 20. Future Improvements

- Add a 16×2 LCD to display moisture readings without a PC

- Integrate a relay and water pump for automatic irrigation

- Replace Arduino Uno with ESP32 for Wi-Fi and remote monitoring

- Send push notifications to a smartphone via MQTT or Blynk

- Log readings to an SD card for historical analysis

- Map ADC value to percentage: moisture% = map(soilValue, 0, 1023, 100, 0)

- Add DHT11 sensor for combined temperature and humidity monitoring

- Power via solar panel and rechargeable LiPo battery for outdoor use

# 21. Safety Precautions

- Verify all connections before powering the Arduino

- Always use 220 Ω resistors in series with LEDs

- Avoid short circuits on the breadboard

- Do not immerse the Arduino board in water

- Clean metal probes after each use to reduce corrosion

- Disconnect power before modifying the circuit

# 22. Conclusion

The Arduino-Based Soil Moisture Monitoring and Alert System demonstrates how a simple resistive sensor and voltage divider can be used to monitor real-world environmental conditions. The sketch uses fundamental Arduino functions — analogRead(), digitalWrite(), Serial.println(), and delay() — making it an ideal first embedded systems project.

The configurable threshold constant and clear code comments make the project easy to calibrate and extend. With straightforward additions such as a relay, pump, or ESP32 upgrade, this system can evolve into a fully automated smart irrigation solution.

Arduino Soil Moisture Monitoring System — Project Documentation v1.0